Table of Contents

Operating Procedures Manual

City of Tampa Job Order Contracting for

Residential Rehabilitation and Repair Services

Bid # 25-P-00039

- Home inspection

- Scope Proposal

- Equipment & Warranty

- Qualifications for subcontractors

- Awarded subcontractors agreement and documents

- Home inspection

- At the beginning of every month 6 homes will be assigned. An email will be sent to us or a phone call.

- Inspection of the houses with will be done with City of Tampa CRA, Healthy homes, and Gordian representatives

  - Planned Dates - Joint Scope Meeting

- Review the items the homeowners have selected to be repaired in the houses

  - Application for Housing Rehabilitation & Renovation Assistance

- Ask owners if there are any other items they need to repair.

- During the home inspection it will be decide which items and area need to be repaired:

  - Do Lead test if built before 1978

  - Do Radon Testing for all healthy homes

  - Other items selected by homeowners or cover during inspection

- Select a day to invite subcontractors to walk the house and provide us with an estimate for every home.

- Scope Proposal

- Prepare an excel spreadsheet (Home Repair Proposal) for each property including:

  - Scope of work

  - All subs estimate

  - subs cost per scope and comparison

- Prepare a scope proposal to be sent to City of Tampa CRA and Healthy homes and Gordian.

- A meeting will be held with the homeowner to approve the proposed cost.

- System procedures: prepare folders to save all documents for every home. Below is the hierarchy and location and folder names:

  - Dropbox: // Lionize Construction

  - 01. Tampa JOC - Home Rehab

  - 01. 2025.  Home Rehab & Repairs Services

  - 01. CRA Homes  - BE

  - 01. Healthy Homes  - Marquaz

    - Walk Through

    - Scopes (COMPLETE HOME REPAIR & REHABILITATION COST CODE MASTER)

    - Quotes

    - Blueprint

    - Test & Permit Fees

    - Documents

- Once the Scope proposal gets approved the date must be posted to Gordian portal in the proposal tab.

- Set up Critical Path Method (CPM) for each home which should be completed in 60 days.

- COMPLETE HOME REPAIR & REHABILITATION COST CODE MASTER organized in the correct Division order (following the CSI Master Format structure):

### 1. Environmental Testing & Hazard Remediation (Division 01-02)

- 01050 – Lead Testing & Inspection (Paint, Soil, Dust)

  - Lead Remediation

    - Provide EPA Certified Lead Abatement Supervisor and workers.

    - Establish regulated area, install barriers, HEPA filtration, and remove lead paint as specified.

    - Perform TCLP sampling and proper disposal.

  - Cost

- 01055 – Asbestos Testing

- 01060 – Mold Testing & Remediation

  - Cost

- 01065 – Radon Testing & Mitigation

  - Cost

- 01070 – Termite Inspection & Reporting

### 2. Site Work (Division 02-31)

- 02200 – ADA Ramps

  - 02210 – Concrete Ramp

  - 02220 – Modular Ramp

  - 02230 – Handrails

  - Cost

- 02600 – Landscaping

  - 02610 – Tree Trimming/Pruning

  - 02620 – Tree Cutting/Removal

  - 02630 – Stump Grinding/Removal

  - 02640 – Brush Clearing

  - Cost

- 02700 – Termite Pre-Treatment (Soil Treatment, Bait Systems)

  - Subterranean Termite Treatment (Annual Renewal)

  - Cost

- 02800 – Termite Damage Repair (Wood Replacement, Structural Reinforcing)

  - Tent Fumigation (Annual Renewal)

  - Cost

- 02800 – Fencing Systems

  - 02810 – New Fence Installation (Wood, Vinyl, Chain Link, etc.)

  - 02820 – Fence Repairs (Posts, Rails, Gates)

  - 02830 – Specialty Fencing (Pool, Pet Safety, Sound Barriers)

  - 02840 – Fence Finishes (Staining, Paint, Anti-Rust)

  - Cost

### 5. Carpentry & Framing (Division 06)

- 06100 – Rough Carpentry (Framing)

- 06200 – Finish Carpentry (Trim, Cabinets)

- 06400 – Millwork (Doors, Windows)

- 06500 – Termite-Damaged Wood Replacement

- 06600 – Structural Reinforcing (Post-Termite Repair)

- Cost

### 6. Roofing (Division 07)

- 07100 – Roof Shingles (Asphalt, Tile, Metal)

- 07200 – Roof Flashing & Sheet Metal

- 07300 – Roof Repairs (Leak Fixes)

### 07500 – Soffit & Fascia Installation/Repair

  - 07510 – Soffit (Ventilated, Aluminum, Vinyl)

  - 07520 – Fascia (Wood, PVC, Metal)

  - 07530 – Painting/Refinishing

  - Cost

### 7. Exterior Finishes (Division 07-09)

- 07500 – Siding (Vinyl, Wood, Fiber Cement)

- 07600 – Stucco & EIFS

- 07700 – Lead-Safe Paint Removal/Encapsulation

- 07800 – Exterior Caulking & Sealing

- Cost

### 8. Doors & Windows (Division 08)

- 08100 – Metal Doors & Frames

  - 08110 – Entry Door Installation

  - 08120 – Sliding Glass Doors

  - 08130 – Security Doors

- 08200 – Wood & Plastic Doors

  - 08210 – Interior Doors

  - 08220 – Exterior Wood Doors

  - 08230 – French Doors

- 08300 – Specialty Doors

  - 08310 – Fire-Rated Doors

  - 08320 – Storm Doors

  - 08330 – Screen Doors

  - Cost

- 08400 – Windows

  - 08410 – Window Replacement

  - 08420 – Picture/Bay Windows

  - 08430 – Skylights

  - 08450 – Glass Replacement

  - Cost

- 08500 – Door & Window Hardware

- 08600 – Door & Window Finishes

- 08000 – Garage Door Services

  - 08010 – Installation (Sectional, Roll-Up)

  - 08020 – Replacement

  - 08030 – Opener Repair

  - Cost

### 9. Drywall & Insulation (Division 09)

- 09200 – Drywall Hanging & Finishing

- 09300 – Tile & Backsplashes

- 09500 – Acoustic Ceilings

- 09700 – Insulation Removal/Replacement (If Contaminated)

- Cost

### 10. Finishes (Division 09)

- 09100 – Ceiling Repairs

  - 09110 – Drywall Patching

  - 09120 – Popcorn Removal

  - Cost

- 09200 – Flooring

  - 09210 – Hardwood Installation

  - 09215 – LVT Flooring

  - 09220 – Tile Repair

  - Cost

- 09300 – Complete Bathroom Remodel

  - 09310 – ADA Features

  - 09320 – Shower/Tub Installation

  - Cost

- 09400 – Complete Kitchen Repair

  - 09410 – Cabinet Refacing

  - 09420 – ADA Mods

  - Cost

### 11. Painting & Finishes (Division 09)

- 09900 – Lead-Safe Interior Painting (EPA RRP Compliance)

- 09910 – Lead-Safe Exterior Painting

- 09950 – Wallpaper & Specialty Finishes

- Cost

### 12. Plumbing (Division 22)

- 22100 – Water Supply Lines

- 22200 – Drainage & Vent Pipes

- 22400 – Fixtures (Sinks, Toilets, Faucets)

- 22600 – Water Heater Repair/Replacement

- 22700 – Lead Pipe Testing & Replacement

- Cost

### 13. Electrical (Division 26)

- 26100 – Wiring & Circuits

- 26200 – Lighting Fixtures

- 26300 – Outlets & Switches

- 26500 – Panel Upgrades

- Cost

### 14. HVAC (Division 23)

- 23100 – Condenser Unit & Air Handler

- 23200 – Ductwork

- 23300 – Thermostats & Controls

- Cost

### 15. Specialties (Division 10-14)

- 10100 – Appliances

- 12300 – Custom Cabinetry

- Cost

- Equipment & Warranty

- HVAC Description: Overview of the system’s features and compatibility.

- Unit Specifications

- Brand: Manufacturer of the system.

- Tons: Cooling capacity (e.g., 2-ton, 3-ton).

- SEER: Seasonal Energy Efficiency Ratio rating.

- Description: Overview of the system’s features and compatibility.

- Primary Components

- Condenser (Outdoor Unit): Exterior component for heat dissipation.

- Heat Pump & Air Handler (Indoor Unit): Interior unit for air circulation and heating/cooling.

- 5KW Electrical Heater/Heat Kit: Optional auxiliary heating element.

- Installation Accessories

- Hurricane Pad: Reinforced base for the outdoor unit.

- Hurricane Straps: Secures the outdoor unit to withstand high winds.

- Hanging Kit: Mounts the indoor unit (e.g., wall brackets).

- Ductwork: Existing or new ducts for airflow distribution.

- UV-Resistant Insulation: Protects ducts/pipes from sun damage.

- Drainage & Overflow Protection

- Primary Drain Pan with Cut-off: Collects condensate and prevents overflow.

- Auxiliary Drain Pan: Backup pan for added leak protection.

- Overflow Safety Switch: Automatically shuts off the unit if water accumulates.

- Electrical Components & Controls

- Electrical Whip: Flexible conduct connecting the unit to power.

- Locking Caps: Seal refrigerant line access ports.

- Programmable Thermostat: User interface for temperature control.

- Refrigeration System Setup

- Reclaim Refrigerant: Safely remove refrigerant from an old unit (if applicable).

- Vacuum & Pressure Test: Ensure no leaks and proper system integrity.

- Safety & Compliance

- Locking Caps: Prevent refrigerant leaks.

- Hurricane Pad: Meet local storm codes.

- Hurricane Straps: Meet local storm codes.

- Overflow Switch: Mitigate water damage risks.

- Warranty

- Labor Warranty: Covers installation workmanship.

- Manufacturer Warranty: Covers parts/defects (e.g., 10-year compressor warranty).

- Comprehensive electrical scope of work organized for a home repair or rehab project, consolidating and categorizing all items provided:

  - Service Upgrades & Main Panel Work

    - Main Panel, Grounding, Meter, and Feeder Upgrades

      - Panel Replacement:

Remove existing panel; install new 100-amp or 150-amp main breaker panel (or 200A Square D QO panel for upgrades).

Install main disconnect and label all circuits clearly.

      - Grounding System:

Drive #_______#_____-foot copper ground rods..

Bond to water/gas piping per code.

      - Meter and Feeders:

Replace meter can/socket with weather-resistant meter base.

Install new service entrance cables (feeders) from meter to panel.

Upgrade overhead mast, rigid pipe, LB, weather head, and service wire if needed.

      - Breakers Included:

100A 2-pole (sub-panel feed), 30A/40A 2-pole (appliances/AC), 20A/15A single-pole (general use).

      - Coordination:

Coordinate with utility company for reconnection.

Pass electrical inspections.

    - 200A Service Upgrade (Specifics)

Replace 200A Square D QO main panel and meter base.

Upgrade grounding system with #4 wire, rods, and clamps.

2. Dedicated Circuits

- Air Conditioning (A/C) Circuit

Install 30-amp or 40-amp dedicated circuit (wire gauge per A/C specs).

Run wire from panel to exterior A/C location with weatherproof disconnect box.

Install matching breaker in panel.

- Water Heater Circuit

Install 30-amp dedicated circuit with 10 AWG wire.

Install double-pole breaker and junction box at water heater.

- Kitchen Circuits

Dedicated 20-amp GFCI circuits for microwave, refrigerator, dishwasher, garbage disposal.

30-amp/40-amp circuit for electric stove/range.

Two GFCI outlets on countertops.

3. Rewiring

- Complete House Rewiring (Excluding Kitchen)

Bedrooms:

15-amp circuits (14 AWG Romex), 2 outlets per wall, ceiling lights, interconnected smoke detectors.

Living/Dining Rooms:

15-amp circuits, ceiling fan/light boxes, dimmer switches.

Bathrooms:

20-amp GFCI circuit, exhaust fan, vanity outlets.

All Rooms:

Remove old wiring, install new circuits back to panel.

Interconnected hardwired smoke detectors with battery backup.

- Kitchen Rewiring

Replace all wiring with 12 AWG Romex.

Dedicated circuits for appliances (see Section 2C).

4. Safety Systems

- Smoke Detectors

Option 1 (No Rewiring): Install 6–8 battery-operated photoelectric detectors (bedrooms, hallways).

Option 2 (Rewiring): Interconnected hardwired detectors with battery backup.

- GFCI Installations

Exterior: Replace #__GFCI outlets with weatherproof covers.

Bathroom: #__GFCI outlet.

Kitchen: #__GFCI outlets with box adjustments.

5. Exterior & Outdoor Work

- Light Fixtures:

Replace exterior fixtures (standard or motion-activated LED floods).

Weatherproofing and wiring included.

- AC Disconnect Replacement:

Replace AC disconnects (labor + materials).

6. Sub-Panel & Specialty Upgrades

- Sub-Panel in Extension Area

Install Square D QO sub-panel (40–60 amp feeder with 2-pole breaker in main panel).

Grounding and labeling included.

- Electrical Demo & Waste Removal

Licensed electricians to remove old/inactive wiring (including main panel cleanup).

Debris disposal and electrical waste recycling.

7. Fixture Replacements

Replace light fixtures (indoor/outdoor).

Includes wiring adjustments and mounting.

8. Compliance & Warranty

All work meets NEC code requirements.

Permits and inspections coordinated.

Labor warranty and manufacturer warranties included.

This Electricity scope ensures clarity, code compliance, and safety while covering all electrical aspects of home rehab.

- Qualifications for subcontractors

- Documents to request qualifications from subcontractors

Documents to request from subcontractors

- Insurance – email the document named (Insurance requirements)

- Request Tampa certification W/MBE and SLBE (if applicable)

- Awarded subcontractors agreement and documents

- Prepare and Send contract agreement

- Prepared and Send form MBD-40 only to Tampa certified companies W/MBE and SLBE